talento smart C25 (classic)

Digital distributor time switch with astro function, 2 channels, integrated Bluetooth,

programming by app, 2TE

Digital distributor time switch with integrated Bluetooth interface and straightforward programming on the device, via the app or on a PC. No additional hardware required for program transfer. Protection against tampering provided by encrypted data transmission, PIN protection and product’s ability to be sealed. Daily, weekly and annual programs with 500 freely combinable switch commands. Shortest switching time 1 second. ON/OFF, pulse, cycle, sunrise/sunset-dependent switch commands, random switch commands.

Supply voltage AC/DC 12/24 V, 50–60 Hz; switching capacity: 2 potential-free changeover contacts with zero crossing detection 16 A/250 V AC; 600 W LED load; 8 years’ power reserve; captive screw terminal with wire protection max. 2.5 mm²; high-resolution dot-matrix display with background lighting; automatic operation, fix ON/OFF, override;

Dimensions: 90 x 35.5 x 60 mm (H x W x D)

Manufacturer: Grässlin

Model: talento smart C25

Item no.: 43.03.0003.1